08/22/2023	CITY OF FONTANA

GENERAL NOTES

FOR

STREET IMPROVEMENT PLANS

1	ALL WORK SHALL BE IN ACCORDANCE WITH THESE PLANS, THE CITY OF FONTANA STANDARD PLANS, THE

CONTRACT PROVISIONS AND THE STANDARD SPECIFICATIONS FOR PUBLIC WORKS CONSTRUCTION ("GREEN

BOOK").  ALL REFERENCE SPECIFICATIONS AND STANDARDS SHALL BE THE LATEST EDITION UNLESS

OTHERWISE NOTED.

2	WHEN A TECHNICAL CONFLICT IS FOUND TO EXIST IN THE CONTRACT DOCUMENTS THAT CAN NOT BE

RESOLVED BY REFERENCE TO  PRECEDENCE PROVISIONS IN THE "GREEN BOOK", THE CONTRACTOR SHALL

IMMEDIATELY REPORT SAID CONFLICT TO THE CITY ENGINEER FOR RESOLUTION.

3	ALL MATERIALS AND METHODS ARE SUBJECT TO THE APPROVAL OF THE CITY ENGINEER.

4	CONSTRUCTION PERMITS SHALL BE OBTAINED FROM THE CITY OF FONTANA COMMUNITY DEVELOPMENT

DEPARTMENT, ENGINEERING DIVISION PRIOR TO THE START OF ANY WORK.   INSPECTION COORDINATION

SHALL BE REQUESTED AT LEAST TWO WORKING DAYS PRIOR TO THE START OF ANY WORK IN PUBLIC

RIGHT-OF-WAY WITHIN THE CITY LIMITS.   CALL (909) 350-7610.

5	THE CONTRACTOR SHALL CONFORM TO ALL TRAFFIC CONTROL POLICIES, METHODS AND PROCEDURES

DESCRIBED IN THE  STATE OF CALIFORNIA  MANUAL OF TRAFFIC CONTROLS, LATEST NON-METRIC EDITION

UNLESS OTHERWISE DIRECTED BY THE CITY TRAFFIC ENGINEER.

6	IT SHALL BE THE RESPONSIBILITY OF THE CONTRACTOR TO MAINTAIN BARRICADES, DELINEATORS OR OTHER

TRAFFIC CONTROL DEVICES AT ALL TIMES.

7	THE CONTRACTOR SHALL OBTAIN A PERMIT TO PERFORM EXCAVATION OR TRENCH WORK FOR TRENCHES 5

FEET OR GREATER IN DEPTH FROM THE CALIFORNIA STATE DIVISION OF INDUSTRIAL SAFETY.

8	THE WALLS AND FACES OF ALL EXCAVATIONS GREATER THAN FIVE (5) FEET IN DEPTH SHALL BE GUARDED BY

SHORING, SLOPING OF THE GROUND OR OTHER APPROVED MEANS PURSUANT TO THE REQUIREMENTS OF THE

DIVISION OF INDUSTRIAL SAFETY OF THE STATE OF CALIFORNIA.  TRENCHES LESS THAN FIVE (5) FEET SHALL

ALSO BE GUARDED WHEN THE POTENTIAL EXISTS FOR GROUND MOVEMENT.

9	NO MATERIAL OR EQUIPMENT SHALL BE STORED IN THE PUBLIC RIGHT OF WAY WITHOUT OBTAINING A

SEPARATE PERMIT FOR THAT PURPOSE.

10	THE LOCATIONS OF UTILITIES SHOWN HAVE BEEN DETERMINED FROM AVAILABLE INFORMATION, HOWEVER, IT

SHALL BE THE RESPONSIBILITY OF THE CONTRACTOR TO DETERMINE, IN THE FIELD, THE TRUE LOCATION AND

ELEVATION OF ANY EXISTING UTILITIES, AND TO EXERCISE PROPER PRECAUTION TO AVOID DAMAGE THERETO.

THE CONTRACTOR SHALL CONTACT UNDERGROUND SERVICE ALERT AT 1-800-227-2600 TWO WORKING DAYS

BEFORE EXCAVATION.

11	THE CONTRACTOR SHALL COORDINATE CONSTRUCTION WITH ALL UTILITY COMPANIES INCLUDING, BUT NOT

LIMITED TO, GAS, TELEPHONE, ELECTRIC,  CABLE TELEVISION, LANDSCAPING, LANDSCAPE IRRIGATION,

DOMESTIC WATER, RECLAIMED WATER, SEWER, STORM DRAIN, FLOOD CONTROL AND CALTRANS.  ALL UTILITY

COMPANIES SHALL BE GIVEN TWO WORKING DAYS NOTICE PRIOR TO WORK AROUND THEIR FACILITIES.

12	THE CONTRACTOR SHALL NOT OPERATE ANY FIRE HYDRANT OR WATER MAIN VALVES WITHOUT APPROPRIATE

AGENCY AUTHORIZATION.   CONTRACTOR SHALL COORDINATE WITH THE APPROPRIATE WATER COMPANY FOR

VALVE OPERATION AND WATER REQUIREMENTS.

13	CURVE DATA REFERS TO THE FACE OF CURB.

14	STATIONING REFERS TO THE CENTERLINE OF STREETS EXCEPT WHERE OTHERWISE NOTED.

15	ADEQUATE CONSTRUCTION CONTROL STAKES SHALL BE SET BY THE ENGINEER TO ENABLE THE CONTRACTOR

TO CONSTRUCT THE WORK TO THE PLAN GRADES.  THE CONTRACTOR SHALL BE RESPONSIBLE FOR THE

PRESERVATION OF BENCHMARKS AND CONSTRUCTION CONTROL STAKING DURING CONSTRUCTION.

16	THE CONTRACTOR SHALL NOT DISTURB EXISTING SURVEY MONUMENTS, MONUMENT TIES OR BENCH MARKS

WITHOUT PRIOR NOTIFICATION TO THE CITY ENGINEER.

17	REMOVAL AND REPLACEMENT OF EXISTING SURVEYCONTROL, INCLUDING SURVEY MONUMENTS, MONUMENT

TIES AND BENCH MARKS,  SHALL BE DONE BY A REGISTERED CIVIL ENGINEER OR LICENSED LAND SURVEYOR.

SURVEY  MONUMENTS THAT WILL BE DESTROYED AS A RESULT OF THIS CONSTRUCTION SHALL BE REPLACED.

THE CONTRACTOR SHALL NOTIFY THE ENGINEER ONE WEEK PRIOR TO CONSTRUCTION SO THAT TIES TO

MONUMENTS CAN BE ESTABLISHED FOR LATER REPLACEMENT OF THE MONUMENT.

18	THE CONTRACTOR SHALL MAINTAIN ACCESS FOR LOCAL RESIDENTS AND BUSINESSES AT ALL TIMES. A

MINIMUM 12 FOOT LANE SHALL BE MAINTAINED AT ALL TIMES IN THE CONSTRUCTION AREA FOR RESIDENTS

AND EMERGENCY VEHICLES.

19	THE CONTRACTOR SHALL PROVIDE AND MAINTAIN AN EFFECTIVE MEANS OF DUST CONTROL, INCLUDING

ADEQUATE WATERING, AT ALL TIMES.

20	ALL GRADING OPERATIONS SHALL BE DISCONTINUED WHEN SUSTAINED WIND VELOCITIES EXCEED 25 MILES

PER HOUR.

21	THE CONTRACTOR SHALL NOT CAUSE ANY EXCAVATED MATERIAL, MUD, SILT OR DEBRIS TO BE DEPOSITED

ONTO PUBLIC OR PRIVATE PROPERTY ADJACENT TO THE RIGHT OF WAY DURING CONSTRUCTION WITHOUT

PRIOR WRITTEN APPROVAL.

22	NO TRENCH BACKFILL SHALL TAKE PLACE WITHOUT PRIOR APPROVAL OF THE CITY INSPECTOR.

23	A GEOTECHNICAL ENGINEER SHALL CERTIFY ALL BACKFILL COMPACTION.  FAILURE TO OBTAIN THE

REQUIRED DENSITY SHALL REQUIRE RE-WORKING OF THAT PORTION OF THE WORK UNTIL THE SPECIFIED

DENSITY IS OBTAINED.

24	CARE SHOULD BE TAKEN TO PREVENT GRADES, DITCHES, AND SWALES FROM UNDERMINING STREET

IMPROVEMENTS.  UPON INSPECTION OF THE SITE, THE CITY ENGINEER MAY REQUIRE TEMPORARY

NON-ERODEABLE SWALES ENTERING OR LEAVING IMPROVEMENTS.

25	THE FINAL LOCATION AND WIDTH OF DRIVEWAY APPROACH APRONS SHALL BE APPROVED AT THE TIME OF

CONSTRUCTION AND SHALL CONFORM TO THE CITY OF FONTANA STANDARD DETAILS.

26	ALL EXPOSED CONCRETE SURFACES SHALL CONFORM IN GRADE, COLOR AND FINISH TO MATCH EXISTING

CONCRETE.

27	THE SEWER CONTRACTOR SHALL STAMP AN "S" IN THE FACE OF THE CURB AT THE LOCATION OF THE SEWER

LATERAL.

28	NO CONCRETE SHALL BE PLACED UNTIL THE FORMS AND REINFORCING STEEL HAVE BEEN PLACED, INSPECTED

AND APPROVED.

29	ALL UNDERGROUND UTILITIES SHALL BE INSTALLED, TESTED AND APPROVED PRIOR TO PAVING OF STREETS.

30	APPROVED SOIL STERILANT IS REQUIRED UNDER ALL NEW ASPHALT PAVEMENT PRIOR TO PLACEMENT.

31	PAVEMENT STRUCTURAL SECTIONS SHOWN ARE MINIMUM AND SUBJECT TO REVISION AND APPROVAL OF THE

CITY ENGINEER AS DETERMINED BY SOILS TESTS TAKEN AFTER COMPLETION OF ROUGH GRADING.

32	ACTUAL THICKNESS OF A.C. PAVEMENT AND/OR BASE COURSE MATERIAL FOR STRUCTURAL STREET SECTIONS

SHALL BE RECOMMENDED BY A GEOTECHNICAL REPORT AND SUBMITTED TO THE CITY OF FONTANA FOR

APPROVAL UPON COMPLETION OF ROUGH GRADING.

33	ALL MANHOLES, CLEANOUT FRAMES, COVERS AND VALVE BOXES SHALL BE RAISED TO FINISHED GRADE BY THE

PAVING CONTRACTOR UPON COMPLETION OF PAVING.

34	UPON COMPLETION OF CONSTRUCTION, CONTRACTOR SHALL RESTORE ALL SIGNING, STRIPING, BARRICADES,

AND OTHER TRAFFIC CONTROL DEVICES TO THE SATISFACTION OF THE CITY TRAFFIC ENGINEER.

35	CONTRACTOR SHALL RELOCATE AND/OR REPLACE LANDSCAPING, SPRINKLERS AND SIDEWALKS AFFECTED BY

THE CONSTRUCTION TO THE SATISFACTION OF THE CITY ENGINEER.

36	AS-BUILT DRAWINGS SHALL BE PROVIDED BY THE CONTRACTOR TO THE ENGINEER OF RECORD, WHO SHALL

PROVIDE RECORD DRAWINGS TO THE CITY ENGINEER.